This is a semi-underground structure.  It is organized around four open rectangular ‘pits’ arranged in a quadrangular (2 x 2) pattern.  The quadrangle is aligned exactly with the cardinal directions. The two eastern pits and their adjacent structures are named “Battery Sanford Kellogg”.  The western pair is named “Battery Benjamin Lincoln”

The description can be divided into four parts:

1)	The Mortar Pits.

2)	The Upper (new) Magazines.

3)	The Lower (original) Magazines.

4)	Protected Switchboard

The Mortar Pits.

There are four pits. The pits as seen mostly date from the 1915-1916 reconstruction

The north pit of Battery Kellogg (Kellogg/”B”) is intact, open, and clear, except for an earthen ramp in  one corner (left for the exit of construction equipment that re-excavated the buried bit in 1992).  Description of the pit:

-	The pit is rectangular, 70’ x 100’, with concrete walls on four sides.

-	The north and south pit walls are 14 feet high; these are the exterior walls of the magazines (see below).

-	The east and west pit walls are approximately 8 feet high; these retain earth fill.

-	The pit is depressed below grade with earth slopes from the top of the concrete walls to the surface level.

-	The concrete floor of the pit shows four circles 20 feet in diameter.  These are the filled and covered mounting platforms for the four 12” breech-loading mortars.

-	This west wall contains a Telautograph data booth. This dates from the original 1892-1896 construction.  The reconstruction was arranged to preserve this feature.  The Telautograph would electrically transmit and instantly reproduce handwritten messages; these messages were firing orders for the mortars

The south pit of Battery Kellogg (“A”) is a mirror-image.  It is completely buried and modern housing has been built over it.

The western pits (Battery Lincoln, “A” and “B”) are generally similar.  However they were only enclosed on three sides and they are open to the west.   Both are partially filled with several feet of earth, paved, and used as parking lots.

- The Upper Magazines.

- These date from the enlargement of the batteries between 1912 and 1916.  There are six magazine traverses.  For each of the two named batteries there are three magazine traverses, one in the center with four rooms, and one on each flank with two rooms each.  Each room is approximately 12’ x 67’.  Except as noted, these are intact. Description:

- -	North Flank, Kellogg:  Two rooms opening onto a corridor/tunnel that connects the north pit with the exterior.  The room adjacent to the pit was for storing projectiles.  The room further to the side was for storing powder.  These rooms served the mortars in the nearest (northern) half of the pit.  This traverse is intact.

- -	Central Traverse, Kellogg:  Four rooms opening onto a corridor that connected the north and south pits.  The two center rooms were for storing powder.  The two side rooms were for storing projectiles.  Each pair of rooms served half of the adjacent pit.  A flight of steps from the corridor accesses the Lower Magazines (see below).  This traverse is intact except that the south end of the corridor is sealed because the south pit is buried.

- -	South Flank, Kellogg:  This is a mirror of the North Flank.  This traverse is buried.

- -	North Flank, Lincoln:  Two rooms similar to the flank traverse of Kellogg.  Because Btry. Lincoln is not enclosed, these rooms opened onto a covered portico with concrete roof supported by columns.  This is traverse intact but the portico is enclosed by concrete block walls between the columns; it is used for storage.  Additional rooms to the side of the traverse are from a powerplant built circa 1915.

- -	Central Traverse, Lincoln:  Four rooms similar to the central traverse of Kellogg.  Due to the existing Lower Magazines (see below), the new magazines could not be placed directly between the pits.  Instead they were ‘flipped’ east-west and extend behind the corridor and pits.  This traverse was extensively modified in WWII; the interior was sheathed with wood.  The interior was later gutted by fire and the interior brickwork is spalling.

- -	South Flank, Lincoln:  This is a mirror of the North Flank without the powerplant.

- Features of the upper magazines:

- Overhead conveyor rails for transporting shells.  These include mechanical switches for selecting between different rails holding different types of shells.

- Linings of porous brick to reduce condensation.

- Built at a floor elevation 4’ higher than the originals to avoid drainage problems.

- The Lower Magazines.

- This complex dates from the original construction of 1892 to 1896. The lower magazines are entirely underground and located between the east and west pairs of pits; they connect the magazines and pits described above.  They consist of a series of concrete galleries and rooms with arched ceilings.  Description:

- -	Two ‘cross galleries’ run north-south.  One connected the pair of pits in each named battery.  These are intact but the original entrances to the pits at the ends of the galleries were sealed during the reconstruction.  Near the ends of these galleries are shallow niches in the walls for fire-control (see below).

- -	From the center of each ‘cross gallery’ a flight of steps leads up to the adjacent upper magazines.

- -	A ‘main gallery’ runs east-west between the ‘cross galleries’.  Near each junction this splits and runs alongside the triangular complex (see below).

- -	Two powder rooms and two storerooms branch off the mid-point of the main gallery at the center of the overall structure.

- -	An irregular niche is cast in the original walls of the main gallery near the powder rooms.  Clay ducts imbedded in the walls lead to other areas of the magazines.  These are part of the original ventilation design (see below).

- -	At the junction of each ‘cross gallery’ with the ‘main gallery’ is a triangular complex of rooms used for ammunition handling (see below).

- Features of the lower magazines:

- This structure contains elaborate interior rooms for ammunition handling.  These additional rooms were an experimental design built only at this battery.

- This structure included provisions for air-conditioning and humidity-control in the original design.  Remnants of these features survive.

- This structure includes special niches and ductwork intended for running cables to the mortars to allow electric firing of all mortars simultaneously.

- The galleries still contain overhead conveyor rails for transporting shells.

- Protected Switchboard

- This is a type of structure known as a ‘Protected Switchboard’ built circa 1920.  It originally contained a telephone switchboard that connected the batteries to remote observation stations and other harbor defenses.

- The Protected Switchboard structure consists of a room (approximately 20’ x 40’) with thick concrete walls and roof built into the side of the embankment.  This provides physical protection from enemy shellfire.  A free-standing wooden building was constructed within this room to house the switchboard and protect it from concussion and condensation.

- Only the entrance at one end protrudes from the embankment.  This was closed by an earthen mound some years ago.  It is currently inaccessible but could be reopened.  The interior is believed to be reasonably intact.

- The Protected Switchboard is physically independent of the mortar battery; they are connected only by communications wires.